SG 9000 - SG S400-20 - Proxy SG 8100 to 9000 Upgrade Document

# Executive Summary

This document will outline the upgrade steps to go from the previous standard (8100) to the new standard (9000).

# Upgrade Steps

Old 8100

- Take Backup of 8100 in 5.4.12.6

- Upgrade 8100 to SGOS 6.2.12.1

- Disable Bridge on 8100

- Take backup of 8100 on 6.2.12.1

**switch over to new hardware

New 9000

- Initial IP/Console configuration

  - Ensure to use admin/admin as the password

  - Delete IPv6 information from interface adapters

  - Ensure interface speed matches switch port speed (standard is hardcoded, not auto)

- Ensure new 9000 is upgraded to 6.2.12.1

- Copy the default keyring certificate and paste into notepad

- Disable bridge on 9000

- From director, edit the device to use SSH user/pw admin/admin

- Restore configuration from 8100 6.2.12.1 to 9000

- Edit the default keyring certificate to the one you have in your notepad

- Disable CPU monitor on 9000

  - Statistics > Advanced > Diagnostics > Stop the CPU Monitor

- Change the restart mode to hardware + software

  - Maintenance > System and Disks > Tasks

- Ensure FTP logs are configured correctly and access logs are receiving data. Protocols should be sent to the correct log as shown below.

- In Maintenance > Service Information > Snapshots delete old serial# job and create new job with new serial#